\subsection{Element Distortion}

An ingredient central to the flexibility of the spectral element method

is the use of (high-order) isoparametric mappings that allow one to

efficiently evaluate, in the computational domain $\Oh := [-1,1]^d$,

derivatives, integrals, and so forth, that are expressed in the physical

subdomain $\Omega^e \subset \Omega$.

We denote the components of the position vectors in physical space

as $\bx=(x,y,z)=(x_1,x_2,x_3)$

and in computational space as $\br=(r,s,t)=(r_1,r_2,r_3)$, as needed.

(The distinction between the time variable, $t$, and the

third component of the reference coordinate, $t$, will

be clear from the given context.)

In the isoparametric formulation, the geometry is represented

with the same basis as the solution variables.

For example, in a three-dimensional element $\Oe$,

\begin{eqnarray} \label{eq:bxr}

\left.  \bx(\br) \right|^{}_{\Oe}

\;&=&\;

\sum_{ijk}

\bx^{e}_{ijk} h_i(r) h_j(s) h_k(t),

\end{eqnarray}

where the sums are taken with $i,j,k \in [0,\dots,N]^3$ and

the $h_i$s are the $N$th-order Lagrangian interpolants based at

the Gauss-Lobatto-Legendre (GLL) nodes $\xi_j$, $j=0,\dots,N$.

The physical mesh coordinates, $\bx^{e}_{ijk}$, are generated

as part of the preprocessing phase.

Typically, the GLL nodes are distributed along the edges in

physical space in an arc-length-preserving fashion and

linear (Gordon-Hall) interpolation is used to import the element

surface/edge deformation into the element interior.

(Further details of the Gordon-Hall mapping are given in Chapter 4

of \cite{dfm}.)

Note that the use of high-order interpolants in (\ref{eq:bxr})

implies that the geometry is represented to high accuracy.

For example, the difference between the arc of a circle and the

distribution given by (\ref{eq:bxr}) will be $\approx 10^{-7}$

for $N=8$ in typical configurations.

Depsite the rapid convergence rate for (\ref{eq:bx}), one still

must adhere to some basic principles in generating the point

distribution $\bx^e_{ijk}$, which governs the isoparatric mapping.

The key idea is that the mapping should be {\em invertible}, which

is to say that the Jacobian of the transformation,

\begin{eqnarray} \label{eq:jacxyz}

J^e(r,s,t)

\;&:=&\;

\mbox{det} \;

\left(

\begin{array}{ccc}

\pp{x}{r}  &  \pp{x}{s}  &  \pp{x}{t} \\

\pp{y}{r}  &  \pp{y}{s}  &  \pp{y}{t} \\

\pp{z}{r}  &  \pp{z}{s}  &  \pp{z}{t}

\end{array}

\right)^{e}

\;=\; \mbox{det} \;\left(\pp{x_i^{}}{r_j^{}}\right)^{e}

\end{eqnarray}

should not vanish on $\Oh$.

Generally speaking, for deformed hexahedral elements, this will be the

case if the edges are smooth and the angles of the edges at the vertices

lie in the open interval $(0,\pi)$.

From a practical standpoint, particularly with respect to

conditioning and accuracy, it is important that the angles be

well away from 0 and $\pi$.

To illustrate the importance of smooth mappings, we consider the

spectral element formulations of the following two-dimensional

examples.  First, consider the (somewhat contrived) bilinear form

\begin{eqnarray} \label{eq:ip1}

(\pp{f}{x},\bb{g}{y}) \;&:=&\; \int_{\Oe} \, \pp{f}{x} \pp{g}{y} \, d\bx,

\end{eqnarray}

which is replaced in the spectral element method by quadrature

\begin{eqnarray} \label{eq:ip1q}

(\pp{f}{x},\pp{g}{y})_N^{} \;&:=&\;

\sum_{i=0}^N

\sum_{j=0}^N

J_{ij}^{e} \rho_{i}^{} \rho_{j}^{}

\pp{f}{x}_{ij}^{} \pp{g}{y}_{ij}^{}

\end{eqnarray}

If $\Oe$ is a rectangle, then $J^e$ is a constant.  Assuming that

$f$,$g \in \PP_N$, then the integrand in (\ref{eq:ip1}) is

in $\PP_{2N-1}$, for which GLL quadrature is {\em exact}.  That is,

(\ref{eq:ip1}) and (\ref{eq:ip1q}) yield precisely the same results.

If $\Oe$ is deformed, then $J^e$ will be a polynomial of degree $M \le 2N-1$

in the 2D case, and $\le 3N-1$ in the 3D case.  Suppose that $M=1$.

Then the quadrature error (the difference between (\ref{eq:ip1}) and

(\ref{eq:ip1q})) will be proportional to the amount of energy in only

the $N$th modes of $f$ and $g$ -- all other modes will be integrated

exactly.   If $f$ represents a numerical solution that we are trying

to compute, then its $N$th-order component will generally be very small.

(Otherwise, the solution is under-resolved.)

uuuu

\documentstyle[11pt]{article}

\def\baselinestretch{1.0}

%\def\baselinestretch{1.0}

\textwidth 6.0in

\textheight 9.25in

\topmargin -0.5in

\oddsidemargin 0.2in

\parindent 5ex

\renewcommand{\topfraction}{.95}

\renewcommand{\bottomfraction}{.95}

\renewcommand{\textfraction}{.05}

%

%  Input the psfig macros...

%

\input psfig

\input pf

\begin{document}

\title{

Spectral Element Solution of Anisotropic Diffusion

in a Tokamak Model Geometry}

\markright{

SEM for Anisotropic Diffusion

}

\author{P. F. Fischer \thanks{

\noindent

Mathematics and Computer Science Division,

Argonne National Laboratory,

Argonne, IL 60439, USA. {\em fischer@mcs.anl.gov}

}

}

\date{\today}

\maketitle

\begin{abstract}

As part of DOE's applications oriented SciDAC project, three

model problems have been proposed by Steve Jardin of PPPL to

test the potential of numerical algorithms for the challenging MHD

problems facing further fusion development work.

The first of these problems is an anisotropic diffusion problem

in a toroidal geometry and is considered in this note.

\end{abstract}

% \keywords{spectral element methods, domain decomposition,

% sparse matrices, parallel algorithms}

% \AMS{65M70,65Y05,65M55}

\vfill

\section{Introduction}

As part of DOE's Center for Extended Magnetohydrodynamics Modeling (CEMM)

SciDAC project, three

model problems have been proposed by Steve Jardin of PPPL to

test the potential of numerical algorithms for the challenging MHD

problems required for further development of fusion technology.

The first of these problems is an anisotropic diffusion problem

in a toroidal geometry.

This report presents implementation details for the spectral element

discretization of the anisotropic diffusion problem and

initial results using the ANL code Nek5000.

The focus here is primarily on the error behavior as a function

of the spatial discretization.  No attempt has been made to adaptively

refine the mesh for these model problems.

Adaptivity is not precluded by the use of the spectral element method

but is not currently supported in Nek5000.

\section{Anisotropic Diffiusion}

We consider the unsteady diffusion problem

\begin{eqnarray}

\label{eq:diff1}

\pp{T}{t} - \nabla \cdot \bK \cdot \nabla T = S

\qquad \mbox{in} \, \Omega,

\end{eqnarray}

where $\Omega$ is the toroidal geometry illustrated in Fig. \ref{fig:torus}.

The major radius of the torus is $R_0$, and the minor radius is $a$.

The temperature, $T$, satisfies homogeneous Dirichlet boundary conditions

on the boundary $\dO$.  The conductivity,

$\bK = \kappa_{||} \btK + \kappa \cI$, is a 3 by 3 tensor comprising

two parts.  The first is the parallel conductivity given by

\begin{eqnarray} \nonumber

\kappa_{||} \btK = \kappa_{||} \frac{\bB \bB}{\;|\bB|^2},

\end{eqnarray}

where $\kappa_{||} $ is the parallel diffusion coefficient,

and $\bB$ is the magnetic vector-field.

The second component, $\kappa \cI$, is the standard isotropic diffusion

term, with $\cI$ the 3 by 3 identity tensor.

Generally, the diffusion is strongly anisotropic, with

$\kappa_{||} \gg \kappa$, which implies that diffusion along

the torus axis is much more rapid than across the torus cross-section.

Typical anisotropy ratios are $\kappa_{||}/\kappa \simeq 10^9$.

The $\bB$-field is specified in a mixture of coordinate systems

that are convenient for toroidal geometries.

For convenience, we list the following relevant vector identity

in polar $(R,\phi,z)$ coordinates:

\begin{eqnarray} \label{eq:id1}

\nabla \times \bB =

\zh \left(

\frac{1}{R} \pp{}{R} ( R \Bp) - \frac{1}{R}\pp{\Br}{\phi} \right) +

\Rh \left(

\frac{1}{R} \pp{\Bz}{\phi}  - \pp{\Bp}{z} \right) +

\phih \left(

\pp{\Br}{z}  - \pp{\Bz}{R} \right) .

\end{eqnarray}

The specified magnetic field comprises two parts:

\begin{eqnarray} \label{eq:bb}

\bB = \bB_0^{} + \nabla \times \alpha \bB_0^{},

\end{eqnarray}

where the axisymmetric field is given by

\begin{eqnarray} \label{eq:b0}

\bB_0^{} =

- \nabla \times \left( \frac{\psi}{R} \phih \right)

+ \frac{1}{R} \phih .

\end{eqnarray}

Applying the vector identity (\ref{eq:id1}) yields for $\bB_0^{}$

\begin{eqnarray} \label{eq:b02}

\bB_0^{} =

\frac{1}{R} \left(

\phih - \zh \pp{\psi}{R} + \Rh \pp{\psi}{z} \right).

\end{eqnarray}

Applying it again to $\alpha \bB_0^{}$ yields

\begin{eqnarray} \label{eq:bc0}

\nabla \times \alpha \bB_0^{}  &=&

\zh \left(

\frac{1}{R} \pp{\alpha}{R} - \frac{1}{R^2}

\pp{}{\phi}  \alpha \pp{\psi}{z} \right)

\\ \nonumber

&-&

\Rh \left(

\frac{1}{R^2} \pp{}{\phi}   \alpha \pp{\psi}{R}

+ \frac{1}{R} \pp{\alpha}{z} \right) +

\phih \left(

\frac{1}{R} \pp{}{z}  \alpha \pp{\psi}{z}

+ \pp{}{R}  \frac{\alpha}{R} \pp{\psi}{R} \right)

\end{eqnarray}

for the second component of $\bB$.

The functions $\psi$ and $\alpha$ are given by

\begin{eqnarray} \label{eq:psi}

\psi(R,z)

\;&=&\;

\Cs \ln \left[ 1 + \frac{r^2}{a^2} \right], \\ \label{eq:alpha}

\alpha

\;&=&\;

\Ca \sin(m\theta - n \phi),

\end{eqnarray}

with constants

\begin{eqnarray}

\Cs := \frac{a^2}{2R_0 q_0}, &&

\Ca := \frac{\alpha_{mn} a^2}{8 R_0 q_0} .

\end{eqnarray}

Here, $\theta$ is the angle in a polar $(r,\theta)$ coordinate

system with origin at $R=R_0$, $z=0$, and $m$ and $n$ are

integers.

We note the following useful identities:

\begin{eqnarray}

\left( \pp{\psi}{z} \zh + \pp{\psi}{R} \Rh \right)

&=& \frac{\Cs}{a} \frac{2 \rt}{1+\rt^2} \rh,

\\

\left( \pp{\psi}{z} \Rh - \pp{\psi}{R} \zh \right) &=&

\frac{\Cs}{a} \frac{2 \rt}{1+\rt^2} \thetah,

\\

\frac{1}{R}

\left( \pp{\alpha}{R} \zh - \pp{\alpha}{z} \Rh \right) &=&

-\frac{m \Ca}{Rr} \cos(m\theta - n \phi) \rh,

\end{eqnarray}

where $\rt := r/a$.

From this, we find for the first term in $\bB$,

\begin{eqnarray}

\bB_0^{}

&=&

x,

\end{eqnarray}

and for the second,

\begin{eqnarray}

\nabla \times \alpha \bB_0^{}

&=&

\left[

\frac{a}{R R_0 q_0}

\frac{n \rt}{1+\rt^2} - \frac{m}{r} \right]

\frac{a^2 \alpha_{mn}}{8 R_0 q_0} \cos(m\theta - n \phi) \rh,

\\ \nonumber

&+&

\left(

\frac{1-\rt^2}{1+\rt^2} - \frac{a}{R} \rt \right)

\left(\frac{1}{1+\rt^2} \right)

\frac{\alpha}{R R_0 q_0} \phih

\end{eqnarray}

There may be a $\cos \theta$ missing here...

Note that, for $\alpha_{mn} = 0$, the $\rh$ component of

$\bB$ vanishes.  The $\bB$-field is oriented on surfaces

parallel to the exterior toroidal surface.

For nonzero $\alpha_{mn}$, we have additional ``varicose''

components of the $\bB$-field that have a nontrivial

$\rh$-component but have no helical ($\thetah$) component.

\section{Spectral Element Discretization}

Our spatial discretization of (\ref{eq:diff1})

is based on the weighted residual formulation

of (\ref{eq:diff1}):

Find $T \in \cH^1_0(\Omega)$ such that

\begin{eqnarray}

\label{eq:var1}

\iO v \pp{T}{t} d\bx

-

\iO

(\nabla v) \cdot \bK \cdot (\nabla T)  d\bx

= \iO v S d\bx

\qquad \forall \; v \in \cH^1_0(\Omega),

\end{eqnarray}

where $\cH^1_0(\Omega)$ is the usual Sobolev space of functions

belonging to $L^2$ whose derivatives also belong to $L^2$.

The essential features of the variational formulation are

that it reduces the continuity requirements on $T$ from

$C^1$ to $C^0$ and that it guarantees a symmetric positive

definite (SPD) linear system for the stiffness matrix (provided

that $\bK$ is SPD).

Discretization by finite or spectral elements amounts to restricting

the search and trial spaces (for $T$ and $v$, respectively) to

finite-dimensional subspaces of $\cH^1_0$, choosing bases for

these spaces, and solving for the basis coefficients such that

(\ref{eq:var1}) is satisfied.

For a Galerkin formulation, one employs the same space and basis

for both the trial and test functions.

\input torus

\input coord

The bases for the spectral element method (SEM) are based on a decomposition

of the domain into $E$ non-overlapping hexahedral elements that may be

isoparametrically mapped to conform to the geometry of interest.

Functions within each element are represented using nodal (Lagrangian)

basis functions based upon the Gauss-Lobatto-Legendre (GLL) quadrature

points \cite{CHQZ}.

Figure \ref{fig:torus} shows a typical mesh for the torus comprising

320 elements of order $N$=8.

The use of hex-based elements is critical to the efficiency of

the spectral element method as it allows functions to be (locally)

expressed in tensor-product form.  For example, on $\Oe$, $e=1,\dots,E$,

one has

\begin{eqnarray} \nonumber

\left.  T(\bx(\br)) \right|^{}_{\Oe}

\;&=&\;

\sum_{ijk}

T^{e}_{ijk} h_i(r) h_j(s) h_k(t),

\end{eqnarray}

where the sum is taken from 0 to $N$ for each index $i$, $j$, and $k$;

$\bx$ is the position vector in $\RR^3$;

$\br$ is the position vector in the reference

domain, $\Oh :=[-1,1]^d$; and

$T^{e}_{ijk}$ is the set of unknown basis coefficients.

The Lagrangian basis functions, $h_i^{}(r)$, satisfy

\begin{eqnarray} \nonumber

h^{}_i(r) \;& \in &\; \PP_N(r) \\ \nonumber

h^{}_i(\xi_j) \;& = &\; \delta^{}_{ij},

\qquad i,j \in \{0,\dots,N\}^2,

\end{eqnarray}

where

$\PP_N(r)$ is the space of polynomials of degree $\le N$;

$\xi_j \in [-1,1]$ is the $j$th GLL point associated with $N+1$

point quadrature; and

$\delta^{}_{ij}$ is the Kronecker delta.

We denote the components of the position vectors

as $\bx=(x,y,z)=(x_1,x_2,x_3)$

and $\br=(r,s,t)=(r_1,r_2,r_3)$, as needed.

(The distinction between the time variable, $t$, and the

third component of the reference coordinate, $t$, will

be clear from the given context.)

In the isoparametric formulation, the geometry is represented

with the same basis as the solution variables, that is,

\begin{eqnarray} \label{eq:bxr}

\left.  \bx(\br) \right|^{}_{\Oe}

\;&=&\;

\sum_{ijk}

\bx^{e}_{ijk} h_i(r) h_j(s) h_k(t).

\end{eqnarray}

The physical mesh coordinates, $\bx^{e}_{ijk}$, are generated

as part of the preprocessing phase.

Typically, the GLL points are distributed along the edges in

physical space in an arc-length-preserving fashion and

linear (Gordon-Hall) interpolation is used to

import the element surface/edge deformation into the element

interior.  Further details may be found in Chapter 4 of \cite{dfm}.

uuu

\subsection{Stiffness Matrix for Anisotropic Diffusion}

In the SEM, the stiffness (or coefficient) matrix is never

explicitly formed.  Rather, since we employ iterative solvers,

we simply require the {\em action} of the stiffness matrix

upon a vector, which is effected by evaluating the local

matrix-vector products in an element-by-element fashion and

assembling the resultant residual vector.

%%% The first step is computationally intensive,

%%% while the second is communication intensive in a parallel setting.

To address implementation of the tensor conductivity, we first

introduce a few of the components that are central to the SEM

matrix-vector multiplication process.

In deformed geometries, extensive use is made of the chain rule

to compute the derivatives that appear in the bilinear forms

of the weighted residual method.

For example, to evaluate the integrand in the second term of

(\ref{eq:var1}), we employ quadrature at the nodal points,

which requires evaluation of derivatives of the form

\begin{eqnarray}

\left.

\pp{T}{x}

\right|^{}_{\bx^e_{mno}}

=

\left(

\sum_{j=1}^d \pp{r_j}{x} \pp{T}{r_j}

\right)^{}_{\bx^e_{mno}}.

\end{eqnarray}

Inserting such expansions into (\ref{eq:var1}) and rearranging terms,

one derives the local stiffness matrix for the

{\em isotropic} diffusion case, given by

\begin{eqnarray} \label{eq:Agen}

A_I^e \; := \;

\bD^T_{} \bG^e \bD \; = \;

\left( \!

\begin{array}{c} D_1^{} \\ D_2^{} \\ D_3^{}\end{array} \! \right)^{\!\! T}

\! \!

\left(

\begin{array}{ccc} G_{11}^{} & G_{12}^{} & G_{13}^{} \\

G_{21}^{} & G_{22}^{} & G_{23}^{} \\

G_{31}^{} & G_{32}^{} & G_{33}^{} \end{array} \right)^e

\left( \!

\begin{array}{c}   D_1^{} \\   D_2^{} \\   D_3^{}    \end{array} \! \right) .

\end{eqnarray}

%% Here, and in the sequel, we drop the elemental-specific

%% identifier (superscript $e$).

The components of the geometric tensor field, $\bG^e$,

\begin{eqnarray}

G^e_{ij} :=

\kappa B^e J^e

\left(

\sum_{k=1}^d

\pp{r_i^{}}{x_k^{}}

\pp{r_j^{}}{x_k^{}}

\right)^{}_{\bx^{e}_{mno}},

\end{eqnarray}

are diagonal matrices comprising the conductivity ($\kappa$), Jacobian ($J^e$),

mass matrix ($B^e$), and metrics ($\pp{r_i^{}}{x_k^{}}$).

The terms $\kappa J^e G^e_{ij}$ are evaluated using point-wise collocation,

as is standard with pseudo-spectral methods.

This is coupled with point-wise quadrature (i.e., $B$ is diagonal), which is

viable because the choice of Gauss-Lobatto-Legendre quadrature points

as nodal points ensures that the quadrature error and approximation

error are of the same order.

The differentiation matrices (in the reference coordinates) are

given by

\begin{eqnarray} \label{eq:Di}

D_1^{} :=  (I \otimes I \otimes \Dh) , \;\;\;\; %\\ \nonumber

D_2^{} :=  (I \otimes \Dh \otimes I) , \;\;\;\; %\\ \nonumber

D_3^{} :=  (\Dh \otimes I \otimes I) ,

\end{eqnarray}

where $\Dh$ is the $(N+1) \times (N+1)$ one-dimensional spectral

differentiation matrix.

The bulk of the computational effort is in applying the derivative operators

(12$N^4$ operations per element).

The bulk of the memory demand is in accessing the $G_{ij}^{}$s

(6$N^3$ entries per element).

Note that if a full mass matrix were used, or if the factored form

(\ref{eq:Agen}) were not employed, the storage and work would

scale as $N^6$, which would be prohibitive for $N > 5$.

%%%

%%%The global stiffness matrix (which is never formed) is formally obtained

%%%by assembling the block-diagonal matrix comprising the local stiffness

%%%matrices, $A_L =

%%%Further details of the SEM formulation may be found in \cite{dfm}.

It is straightforward to incorporate a full diffusion tensor, $\bK$, into

the framework introduced above.

To begin, we expand the second integrand on the left of

(\ref{eq:var1}) to obtain

\begin{eqnarray} \nonumber

\nabla v \cdot \bK \cdot \nabla T

\;&=&\;

\sum_{i,j} \pp{v}{x_i} K_{ij} \pp{T}{x_i}

\\ \nonumber \;&=&\;

\sum_{l,m} \sum_{i,j} \pp{v}{r_l}

\left( \pp{r_l}{x_i} K_{ij} \pp{r_m}{x_i}  \right)

\pp{T}{r_m},

\end{eqnarray}

where the summation indices range from 1 to $3$.

Inserting the local basis functions and applying

quadrature, the local stiffness matrix for a single element is

\begin{eqnarray} \label{eq:dgdt}

A^e \; := \;

\bD^T \bGt^e \bD

\end{eqnarray}

where  $\bD := (D_1, D_2, D_3)$, and $\bGt^e :=$block($\Gt^e_{lm}$), with

\begin{eqnarray} \nonumber

\Gt^e_{lm}

=

B^e J^e

\left(

\sum_{i,j}

\pp{r_l^{}}{x_i^{}}

K_{ij}

\pp{r_m^{}}{x_j^{}}

\right)^{}_{\bx^{e}_{mno}}.

\end{eqnarray}

Note that the structure of (\ref{eq:dgdt}) is identical to

(\ref{eq:Agen}).  Only the entries in $\bG^e$ and $\bGt^e$ differ.

Because the coefficients $\Gt_{ij}$ are

computed in a preprocessing step, the cost of applying the

stiffness matrix in the case of the tensor conductivity

is {\em identical} to the cost of applying the standard

SEM stiffness matrix.

\subsection{Meshing the Toroidal Geometry}

\label{sec:mesh}

We meshed the geometry by constructing standard

decompositions for cylindrical geometries and then analytically

wrapping these into the desired toroidal configuration.

A typical three-dimensional configuration is illustrated

in the isometric and cut-away views of Fig. \ref{fig:torus}.

Four cylindrical cross-sections were considered and

are shown in Fig. \ref{fig:cylcs}.  Mesh (c) was obtained

as a quad-refine of (b), and (a) was a result of coarsening (b).

%%  Mesh (d) employed concentric rings of elements, save

%%  for the four elements on the cross-point, which were (extruded)

%%  straight-sided quadrilaterals.

In the toroidal direction, we used 8, 16, 32, and 40

levels of elements for Meshes (a), (b), (c), and (d), unless otherwise

noted.

The Nek5000 preprocessor runs in single precision and thus

generates only 7 significant digits in the input mesh.

For some of the more precise computations this had an

impact on the final error, so we mapped all boundary

vertices to the unit cylinder prior to applying the

toroidal wrap.   The solver runs in 64-bit precision

so this step eliminates that as a source of significant error.

\input cylcs

\section{Evolution from Steady State}

In the initial problem, we consider the case $\kappa=1$, $\kp=10^k$,

$k=0,2,4,6,$ and 8.  The source term is chosen such that, at steady

state, the temperature has a parabolic profile given by $T = 1 - (r/a)^2$.

Starting with this profile as an initial condition, we seek to integrate

the solution for long times and observe the deviation of the centerpoint

temperature from unity.   Since the initial condition is equal to the steady

state solution, this amounts to assessing the deviation of the initial

residual from zero.   Also, because of the particular nondimensionalization

($\kappa=1$, $\kp=10^k \gg 1$), it is clear that we will need an implicit time

integrator to reach time $t \ge 1$, and thus we will need to invert systems

of the form $H := c/\dt B + A$, where $c$ is an order-unity constant

associated with the choice of timestepping scheme.

Note that, for $\kp/\kappa \longrightarrow \infty$, the PDE has a nontrivial

kernel that admits {\em any} function that depends only on $r$ as a solution

to the steady state homogeneous problem.  As a consequence, we expect

the (steady state) stiffness matrix, $A_{\infty}$, for the discrete problem

to be singular in this limit.

\subsection{Steady State Problem}

Prior to solving the unsteady problem, we first consider the steady state

problem, which avoids issues of temporal accuracy and allows a detailed

survey of the behavior of the spatial discretization.

The solution is obtained

using Jacobi preconditioned conjugate gradients (PCG) to solve

\begin{eqnarray} \label{eq:ss1}

A {\underline \delta T} = S - A {\underline T^{\infty}},

\end{eqnarray}

where ${\underline T^{\infty}}$ is the exact steady state solution (and the initial

condition, in this case).   Note that this is a very common approach

to solving steady state problems as it allows one to incorporate any

inhomogeneities in the boundary conditions into the right-hand side,

such that ${\underline \delta T}$ satisfies homogeneous boundary

conditions only.

Table \ref{tab:prob1a} shows the deviation from unity of the

steady state centerpoint temperature value for each of the four

meshes shown in Fig. \ref{fig:cylcs}. Several values of polynomial

degree $N$ and parallel conductivity $\kp$ are considered.

We see that, in all cases, spectral convergence is obtained with

increasing polynomial order $N$.

Moreover, for the nearly isotropic case ($\kp=10^0$), classic second-order

convergence is observed for $N=1$, as can be inferred from

the upper-left entries for each of cases (a), (b), and (c).

No such convergence is observed for large values of $\kp$.

The principal conclusion to draw from Table \ref{tab:prob1a} is that

the low-order cases ($N \le 3$) have significant difficulties in maintaining

a low error as $\kp$ increases, while the high-order cases do not suffer from

this difficulty at all.

Note that standard adaptive meshing approaches based on low-order

discretizations would not materially improve the situation because the

solution here is uniformly smooth and devoid of ``hot spots'' where

mesh adaptivity would prove effective.

Thus, for this problem,

discretizations involving a few high-order elements are preferable

over those having a larger number of low-order elements.

This point is illustrated in Fig. \ref{fig:lovhi}, which shows the

steady-state centerpoint error versus the number of

degrees-of-freedom for Meshes (a)--(c).  The parallel conductivity

is $\kp = 10^8$.

We remark that, in order to achieve an error $< 10^{-7}$, we had to modify

the input cylinder geometry to ensure that the boundary vertices were at unit

distance from the origin to within machine precision.  In Nek5000 the input

geometry is currently specified only to 7 digits.  For the (rare) cases where

greater precision is required, a user-defined subroutine is available to

adjust the input data prior to computation of the Jacobian and metrics

associated with the isoparametric mapping.   (We occasionally use a

subparametric mapping in which $\bx \in \PP_{N-2}^{}$.  This is related to

enforcement of the divergence-free constraint for Navier-Stokes

discretizations based on the consistent $\PN2$ formulation.)

\input prob1a_tab

\input lovhi

There are some other important issues that are evident from Table \ref{tab:prob1a}.

Examining $\kp=10^8$

column for Mesh (a), one observes a $> 10^4$ {\em increase} in error

as the polynomial degree is increased from 4 to 5, and a similar behavior

in going from $N=6$ to 7.

At first sight, this might indicate a lack of robustness in spectral methods.

Even-odd convergence parity has been observed in other spectral and Galerkin

formulations (see, e.g., \cite{ca88,fm01} for a discussion).

The difficulty here, however, is in fact deeper and is intrinsic to the

anisotropic diffusion problem in this particular geometry.

First off, as a point of reference, we note that the PCG iteration did

not converge for several of the Mesh (a) cases because of the nearly

singular nature of the stiffness matrix in the large $\kp$ limit.

However, PCG did converge for every other mesh and for any Mesh (a)

case where $N$ was even.

The common feature of all of the unsuccessful

cases is that the mesh did not contain the centerpoint and

the interpolation error therefore did not vanish at the centerpoint.

As a consequence, the initial residual had a nontrivial component

in the null-space of $A_{\infty}$ which could not be annihilated by PCG.

By contrast, for any mesh that contained the centerpoint, PCG converged

without difficulty because the initial residual was effectively orthogonal

to ker($A_{\infty}$).   Note that, while the singular nature of $A_{\infty}$

might appear to be inconsequential in the time-dependent case where one must

solve problems in $H$, we will typically be interested in taking rather

large time steps, in which case $H \longrightarrow A$, and the problem

persists.

%% The singular nature of $A_{\infty}$ points to another potential difficulty

%% regarding the centerpoint temperature.  If $\kappa = 0$, then the temperature

%% on the torus axis ($R=R_0$) is governed by a one-dimensional diffusion .

%% problem. This is a set of measure zero with respect to the

%% two-dimensional toroidal surfaces where the other (independent)

%% diffusion problems

%% MORE HERE

\subsection{Time Dependent Problem}

\input errus

To illustrate the unsteady behavior, we have selected two cases from Table

\ref{tab:prob1a} and discretized these in time using a third-order backward

difference formulation (BDF3).   The solution was advanced for 2000 steps

and the timestep size was increased by 10 percent

on every step, starting with an initial step size of $10^{-9}$ and ending with

a maximum step size of $10^9$.  The mesh was employed 8 layers of elements

in the $\phih$ direction, with 5 elements in each cross-section, configured

as shown in Fig. \ref{fig:cylcs} (a).  The maximum pointwise error versus

time is shown in Fig. \ref{fig:errus} for polynomial degrees $N=4$ and $6$.

Spectral convergence with $N$ is clearly obtained.   Slight changes in the

timestep size (e.g., scaling $\dt$ by 1.2 instead of 1.1 at each step) had no

significant effect on the error behavior.   This geometric scaling approach

to selection of timestep size

was developed for the two-dimensional model problems of the next section and

was found to be nearly optimal.

We are currently investigating implicit Runge-Kutta schemes with error control

in order to automate the timestep selection.

\section{Evolution of a Gaussian Pulse}

\input temp1a

The second problem represents a truly unsteady problem in that one

begins with an initially isolated Gaussian pulse in the temperature

field that rapidly spreads out on the conducting toroidal surfaces

within the geometry.  The perpendicular diffusivity (in the interior

radial direction) is assumed to be zero, so energy on any given toroidal

surface should be conserved.  This can be monitored by computing the

integral

\begin{eqnarray} \label{eq:tint}

\langle T \rangle (r,t) =

\int_0^{2 \pi} d \theta

\int_0^{2 \pi} d \phi  R T

.

\end{eqnarray}

Because of the tools available in Nek5000, we decompose the integration

into two steps.  The first is the integration in the $\phih$ direction,

which is done within the solver.  This yields a two-dimensional field

in the torus cross-section.   Subsequent integrations in the $\thetah$

direction are done in the post-processor.   Presently, all Nek5000 output

is in 32-bit precision, so we can only hope for $\approx 10^{-7}$ accuracy

in the final result.

Figure \ref{fig:temp1a} shows the $\phi$-integrated temperature distribution

at different times.   There is no visible diffusion in the radial

direction.

\subsection{Two-Dimensional Model Problem}

\input sol2d

Before proceeding with a detailed quantification of the diffusion in

the toroidal geometry, we first consider a simpler two-dimensional

model problem that has an exact solution in order to more readily

isolate spatial, temporal, and integration errors.

Consider (\ref{eq:diff1}) in the periodic domain $\Omega := [0,1]^2$,

with $\bB$ a uniform

vector field oriented at an angle $\beta$ relative to the

positive $x$-axis.  Let $\kappa_{||}=1$ and $\kappa=0$, which implies

that diffusion will only occur along lines parallel to $\bB$.

Let the initial temperature distribution be given by

\begin{eqnarray}

\label{eq:ic2d}

T(0,x,y) =

[ \sin( \pi x )

\sin( \pi y ) ]^{40},

\end{eqnarray}

which is similar to a two-dimensional Gaussian distribution.

The exact solution to (\ref{eq:diff1} is

\begin{eqnarray}

\label{eq:ic2dexact}

T_{ex}(t,x,y) =

\sum^{m/2}_{\stackrel{l \; =}{-(m/2-1)}}

\sum^{m/2}_{\stackrel{k \; =}{-(m/2-1)}}

\hT_{kl} e^{-i 2 \pi (kx+ly)} e^{-\lambda_{kl}t},

\end{eqnarray}

where

\begin{eqnarray} \nonumber

\lambda_{kl} = c^2 k^2 + s^2 l^2 + 2cskl

\end{eqnarray}

are the eigenvalues and $\hT_{kl}$ the coefficients

associated with the Fourier transform of the initial condition.

Here, $c := \cos(\beta)$ and $s := \cos(\beta)$ and $m=20$ is the

maximum nontrivial wavenumber in the initial condition.

Note that certain combinations of $c$, $s$, $k$, and $l$ can result

in $\lambda_{kl}$ being zero or very small even for $kl \ne 0$.

For example, for $\beta=77$ degrees, the smallest nontrivial eigenvalue

pair has magnitude $\approx$ 6.20e-5, while the second smallest has

magnitude $\approx$ 0.212.  The largest is $\approx$ 2.27e4.

Convergence studies were carried out for the case $\beta = 77$ degrees,

which is a configuration for which the diffusion does not (immediately)

wrap back onto itself.

Figure \ref{fig:sol2d} shows typical isotemperature fields at early times.

Figure \ref{fig:err2d} shows the spatial convergence for several spectral

element configurations consisting of an array of $E=E_1 \times E_1$

elements of order $N$.  Here, $\cN = E_1 N$ represents the number of gridpoints

in each of the $x$ and $y$ directions.

We see that, {\em for fixed resolution}, $\cN$, it pays in every case

to increase the polynomial degree.  The savings is particularly

significant for larger values of $\cN$, where the solution is fully

resolved.

The {\em cost} is essentially dependent on $\cN$ and only weakly

dependent on $N$.  For example, for $\cN=128$, the wall clock times

to reach a final time of $t=10$

were 4232, 3623, and  4659 seconds for $N=2$, 4, and 8, respectively,

while for $\cN=64$ the times were

1691, 667, 526, and 588 seconds for $N=1$, 2, 4, and 8, respectively.

%%% (Our SE implementation is not highly optimized for low $N$.)

\input err2d

The error (in space) was computed as follows.  The spectral element

solution was interpolated onto a regular $\cN \times \cN$ array of

points $(\eta_i,\eta_j)$, $\eta_i = (i-1/2)/\cN$, $i=1,\cN$.  This

point set prevents coincidence with the SEM nodal point sets,

particularly

for the cases $N=1$ and 2, so nodal-superconvergence is avoided.

The initial condition was sampled on this point set, the FFT

was used to find $\hu_{kl}$, and (\ref{eq:ic2dexact}) was used

to evaluate $T_{ex}$.  The maximum absolute value of the difference

between these two solutions was then computed and the result was

divided by $\max | T_{ex} |$ to obtain the relative error.

For $\cN < 40$, the number of sample points was doubled in each

direction.   This is sufficient to ensure that the Fourier spectral

error is zero (to machine precision) in the case $\cN=32$.  In the

$\cN=16$ case it was assumed that the spatial error of the SEM

would dominate the spectral error.

Because of the wide range of temporal scales, achieving a small

temporal error required some care, particularly for the

$(E_1,N)=(8,16)$,

case, which has an error ranging from $10^{-11}$ to $10^{-9}$.

Initially, the pulse experiences a rapid decay of the high-wavenumber

components, which requires a very small timestep to be accurately

captured.  Subsequent decay occurs over much longer time scales.

We employed a third-order backward difference formula (BDF3) in time

that is initialized with BDF1 and BDF2 on the first and second time

steps.  The initial steps were taken to be very small ($10^{-7}$ in

all cases except the (8,16) case, for which it was $10^{-10}$).

The step size was increased geometrically by a factor of 1.05 over

the first 100 steps.  Subsequent to this, it was increased at a rate

that would yield a 10-fold increase in step size every 2000 steps.

This approach was found to be nearly optimal.   Taking too small

a step throughout the calculation (e.g., $\dt=10^{-7}$) was subject

to round-off errors that were noticeable in the (8,16) case.

\subsection{Three-Dimensional Unsteady Problem}

We return to the three-dimensional toroidal geometry of

Fig. \ref{fig:torus} and consider an initial temperature

distribution given by

\begin{eqnarray} \nonumber

T(0,\bx) = e^{-[(R-R^{}_1)^2-(R^{}_1 \phi)^2-z^2]/\delta^2}.

\end{eqnarray}

Here, the major and minor radii are $R_0=3$ and $a=1$,

while $R^{}_1 := 3.5$ and $\delta=0.1$.

The conductivities are $\kp=1$ and $\kappa=0$.

\input errhisp2

Because $\kappa=0$, the energy along any toroidal surface

($r=$constant) should be conserved  and $ \langle T \rangle $ given by

(\ref{eq:tint}) should thus be time invariant.

Figure \ref{fig:errhisp2} (a) shows plots of $ \langle T \rangle $ versus

$r$ for several (unevenly spaced) time points in the interval

$t\in [0,450]$ while

Figs. \ref{fig:errhisp2} (c) and (d) show the departure from the

initial condition, given by

$| \langle T \rangle (r,t)- \langle T \rangle (r,0)|$,

for an $E=20 \times 9$ mesh with $N$=13

(389493 dofs) and $15$ (599535 dofs), respectively.

The meshes are based on the cross-section of Fig. \ref{fig:cylcs}(b).

The timestepping was BDF3 with

an initial timestep size of $10^{-6}$ and final step size

of $10^{-2}$.

The $L^2$ error histories are

shown in Fig. \ref{fig:errhisp2}(b).

The histories indicate that the relative error is growing, but

still $< 2\times 10^{-5}$ out to time $t=450$ for $N$=13,

while it is relatively flat and well below this level for $N$=15.

Figures \ref{fig:errhisp2}(c) and (d) reveal that the $L^{\infty}$

error is governed by the solution at $r=0$.   This might be expected,

given that the centerpoint temperature is strongly influenced by

the coupling of the basis functions in the radial direction and

that, in the weighted residual formulation, the ``weight'' associated

with the $r=0$ point is very small.   It's conceivable that one could

control this by using a weighting function in the weighted

residual formulation that scales as $1/r$.   However, for such

an approach to be effective, it would be necessary to use a full

mass matrix.

The incorporation of an additional weight function in

conjunction with a diagonal mass matrix would simply be equivalent

to multiplying both sides of the equations by a diagonal matrix

and would yield the desired result.

It's not clear that the use of a full mass matrix

would be beneficial in the present case.

It also seems that the inverse-radial scaling would

need to be modified in the general context.

As a closing remark, we comment that the integration in $\theta$

was done using the trapezoidal rule with 128 points.  For periodic

functions the trapezoidal rule corresponds to Gaussian quadrature

and therefore converges exponentially fast with the number of sample

points.  A comparison of 64 and 128 points indicates that 128 points

is sufficient for the present case.

In addition, $\theta$ integration was carried out in the Nek5000

postprocessor, which reads the solution in only 32-bit precision.

Thus, this particular interrogation is limited to about 7 digits

of accuracy.   Clearly, more precision could be obtained by computing

the final integration inside the solver in 64-bit arithmetic or

by storing the output of the solver in 64-bit precision.

An additional source of error in the integration is the interpolation

that is required to transfer the data from the SEM gridpoint layout

to the regular $(r,\theta)$ grid.  Off-gridpoint interpolation in

deformed geometries requires the use of Newton's method.  For, while

$\bx(\br)$ is a known polynomial in $\br$ (see (\ref{eq:bxr})), the

inverse, $\br(\bx)$ is not known analytically.  Again, since the

geometry in the Nek5000 postprocessor is only evaluated in 32-bit

precision, expected accuracy is limited.

\section{Simulation of Magnetic Islands}

%% \section{Appendix A}

\end{document}

n = nx1*ny1*nz1*nelt

do i=1,n

c

rr = rr0(i,1,1,1)

ph = phi(i,1,1,1)

y  = ym1(i,1,1,1)

r  = rra(i,1,1,1)

c  = cos(ph)

s  = sin(ph)

c

scale = a0*a0/( (a0*a0+r*r)*rr*R0*q0 )

c

rrb = -scale*y        ! B0 field

ppb =  1/rr

yyb =  scale*(rr-R0)

c

xxb = c*rrb - s*ppb

zzb = s*rrb + c*ppb

vx(i,1,1,1) = xxb     ! Put B0 field into velocity,

vy(i,1,1,1) = yyb     ! just for diagnostics

vz(i,1,1,1) = zzb

c

kappa_p = pk

bb      = xxb*xxb + yyb*yyb + zzb*zzb

scale   = kappa_p/bb

c

kxy(i,1) = scale*xxb*xxb + kappa_i

kxy(i,2) = scale*yyb*yyb + kappa_i

kxy(i,3) = scale*zzb*zzb + kappa_i

kxy(i,4) = scale*xxb*yyb

kxy(i,5) = scale*yyb*zzb

kxy(i,6) = scale*zzb*xxb

c        write(29,2) i,x,y,z,rr,r,(kxy(i,l),l=1,6)

enddo

c     call exitt